Digital Design Preliminary Exam

(Spring 2018)

Problem 1 		20 points

Problem 2		20 points

Problem 3		15 points

Problem 4		15 points

Problem 5		15 points

Problem 6		15 points

Total			100 points

Name and Student ID:  _______________________________________________________

Name	(Print Please)			Student ID

No Calculators allowed.

Problem 1

Simplify the following function, f= A’B’C’D’ + A’B’C + ACD + ABC’D + AB’C’D using a K-map.

Problem 2

Design a sequential counter which counts the following sequence in the order listed: (0,1,2,5,7,0). The sequence starts at zero and ends at zero.  Implement with a T-flip flop or D-flip flop.  Specific the flip flop you will use.
Note:  Unused states are don’t’care conditions.

- Illustrate the State Table

- Illustrate the State Diagram

- Draw the Sequential Circuit

Problem 3

Implement the following functions using a) Programmable Logic Array (PLA) and b) Programmable Array Logic (PAL).  Illustrate the Programming Tables.

F1 = A’BC + A’B’C’ + AB’C

F2= AB + A’B’C’D’

Problem 4

Design a 4 to 1 multiplexer to implement the following function:

F(w,x,y) = w’x’y’ + wxy + wxy’ + w’xy

Draw the multiplexer in block form.

Problem 5

Using Block Diagrams, illustrate the fundamental differences between the RAM and ROM memory modules.  Please label all inputs and outputs.

Problem 6

Perform the following mathematical operation using 2’s complement. Both numbers are unsigned.

01000101 – 00111010